PixelFlow: Final Major Project Presentation Script & Viva Defense Guide

Yeshwantrao Chavan College of Engineering | Department of Electronics Engineering (VLSI Design & Technology)
Project Group Members: 1. Ajay Rathod  2. Harshita Uikey  3. Varad Vyawhare  4. Sujal Chinche

PART 1: Image Acquisition & Hardware Preprocessing Pipeline

Hardware Pipeline Overview: In a real-world FPGA VLSI system, image acquisition occurs through a 5-step hardware pipeline:

1. Sensor / Camera Capture: A digital CMOS camera (e.g. OV7670 or HDMI receiver) captures a 2D image matrix of size M x N pixels.
2. Color Channel Demuxing: The raw 24-bit RGB image is separated into three independent 8-bit color matrix channels: Red (R), Green (G), and Blue (B).
3. Hex Memory File Generation (.mem): FPGA Block RAMs (BRAM) require initialization data in Hexadecimal format ('00' to 'FF' for 8-bit uint8 values 0 to 255). Python converts pixel values into 2-digit hex strings stored in 'hdl/sunflower_input_32x32.mem'.
4. FPGA Block RAM Initialization: Inside Verilog testbench 'tb_image_filter.v', the $readmemh system task loads hex memory files directly into memory registers.
5. Streaming Row-by-Row Interface: Every clock cycle (10 ns), the testbench reads 1 complete row of N pixels (e.g. 32 x 8 = 256 bits for 32x32 image) and streams it into 'image_filter_top.v' on bus row_in.

PART 2: Bit-Width Rationale & Precision Analysis

Bit-Width Design Rationale: To prevent arithmetic overflow while minimizing FPGA logic resource utilization, exact bit widths were chosen as follows:

| Signal Name | Bit Width | Value Range | VLSI Design & Precision Rationale |
| --- | --- | --- | --- |
| row_in (Input Pixels) | 8 bits / pixel | 0 to 255 (0x00-0xFF) | Standard 8-bit unsigned integer per color channel (2^8 = 256 intensity levels). |
| v_row / u_col (Weights) | 16 bits signed | -32,768 to +32,767 | Filter coefficients ([-1, +3, -1]) require signed numbers for negative spatial derivatives (edge detection). |
| rconv_out (Intermediate) | 20 bits signed | -524,288 to +524,287 | Accumulates 3 products: sum(255 * 3) = 2,295. 20-bit signed registers guarantee ZERO arithmetic overflow. |
| delay_regs (Line Delays) | 20 bits / pixel | Intermediate RConv values | Stores entire 20-bit row outputs for K-1 clock cycles inside FPGA BRAMs / Shift Registers. |
| row_out (Filtered Output) | 8 bits / pixel | 0 to 255 | Final output is scaled down (divided by kernel normalization factor 4 via >>> 2) and clipped to 8-bit unsigned range. |

PART 3: Line-by-Line Verilog Code Snippets & Explanations

3.1 rconv_array.v — Direct-Form 1D Row Convolver

module rconv_array #(
    parameter N = 8,          // Image width (N pixels per row)
    parameter K = 3,          // Kernel size (3x3 filter)
    parameter PIXEL_WIDTH = 8 // 8-bit unsigned pixels
)(
    input  wire                      clk,
    input  wire                      rst_n,
    input  wire                      in_valid,
    input  wire [N*PIXEL_WIDTH-1:0]  row_in,
    input  wire signed [15:0]        v_weights[0:K-1],
    output reg                       out_valid,
    output reg  signed [19:0]        row_out[0:N-1]
);

Module Header Explanation: Unpacks 1D bus row_in into N individual 8-bit pixel signals, inputs 3 signed 16-bit filter weights v_weights ([-1, +3, -1]), and outputs N 20-bit signed accumulation values row_out.

// Unpack 1D packed input row into 2D array and pad boundaries with 0
    wire signed [16:0] padded_row[0:N+K-2];
    genvar p;
    generate
        for (p = 0; p < K/2; p = p + 1) begin : pad_left
            assign padded_row[p] = 17'sd0;
        end
        for (p = 0; p < N; p = p + 1) begin : unpack
            assign padded_row[p + K/2] = $signed({1'b0, row_in[p*PIXEL_WIDTH +: PIXEL_WIDTH]});
        end
        for (p = 0; p < K/2; p = p + 1) begin : pad_right
            assign padded_row[N + K/2 + p] = 17'sd0;
        end
    endgenerate

Boundary Padding Explanation: Pads left and right boundaries with 0 (Zero-Padding boundary condition) and converts 8-bit unsigned pixels into 17-bit signed values ($signed({1'b0, pixel})) for safe signed arithmetic.

integer c;
    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            out_valid <= 1'b0;
            for (c = 0; c < N; c = c + 1) row_out[c] <= 20'sd0;
        end else begin
            out_valid <= in_valid;
            if (in_valid) begin
                for (c = 0; c < N; c = c + 1) begin
                    row_out[c] <= (padded_row[c] * v_weights[0]) +
                                  (padded_row[c+1] * v_weights[1]) +
                                  (padded_row[c+2] * v_weights[2]);
                end
            end
        end
    end

Parallel MAC Loop Explanation: On every posedge clk, computes N parallel row dot-products in 1 single clock cycle with 0 line buffers.

3.2 cconv_array.v — Transpose-Form 1D Column Convolver

reg signed [19:0] delay_regs[0:K-2][0:N-1];
    integer stage, px;
    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            out_valid <= 1'b0;
            for (stage = 0; stage < K-1; stage = stage + 1)
                for (px = 0; px < N; px = px + 1) delay_regs[stage][px] <= 20'sd0;
        end else begin
            out_valid <= in_valid;
            if (in_valid && cconv_en) begin
                for (px = 0; px < N; px = px + 1) begin
                    delay_regs[0][px] <= row_in[px] * u_weights[0];
                    delay_regs[1][px] <= delay_regs[0][px] + (row_in[px] * u_weights[1]);
                    row_out[px] <= delay_regs[1][px] + (row_in[px] * u_weights[2]);
                end
            end
        end
    end

Transpose Line Delay Explanation: Instead of storing 2 full rows before computation, Transpose-Form places line delays Z^-N between multiplier stages. This produces the FIRST complete output row at Cycle 5, achieving ultra-low fixed latency!

PART 4: Slide-by-Slide Word-for-Word Presentation Speaking Script

Slide 1: Title & Group Introduction

Speaking Script: Good morning respected guide, external examiners, and panel members. We are group members Ajay Rathod, Harshita Uikey, Varad Vyawhare, and Sujal Chinche from the Department of Electronics Engineering (VLSI Design & Technology) at Yeshwantrao Chavan College of Engineering. Today we present our final year major project titled PixelFlow: A Low-Latency Feed-Forward Architecture for Image Processing.

Slide 2: Problem Statement & Latency Breakthrough

Speaking Script: In real-time computer vision applications, 2D spatial image filtering is a critical bottleneck. Traditional pixel-by-pixel architectures read 1 pixel per clock cycle and must store (K-1)*N pixels in heavy line buffers before producing the 1st output pixel. For a 512x512 image, traditional architectures waste 1,024 clock cycles. Our PixelFlow architecture processes an ENTIRE ROW of N pixels simultaneously in 1 cycle, producing the 1st output row in just 5 clock cycles—reducing total processing latency by over 500 times!

Slide 3: Micro-Architecture Design (RConv + CConv)

Speaking Script: PixelFlow divides 2D separable convolution into two cascaded 1D systolic arrays: RConv Array (Direct-Form FIR) computes horizontal row dot-products in parallel with 0 line buffers, while CConv Array (Transpose-Form FIR) cascades column multiplication through K-1 inter-stage line delays.

Slide 4 & 5: Visual Evaluation Results (flower & sunflower)

Speaking Script: We evaluated PixelFlow across three image resolutions—32x32, 256x256, and 512x512—for two test images: flower.png and sunflower.png. PixelFlow produces pixel-exact edge enhancement and spatial sharpening identical to software reference models.

Slide 6: Latency Scaling & Performance Comparison

Speaking Script: For Low-Res 32x32, PixelFlow achieves a 29.5x speedup. For Med-Res 256x256, PixelFlow achieves a 253.1x speedup. For High-Res 512x512, PixelFlow completes the frame in 517 cycles compared to 263,170 cycles for traditional architectures—achieving a 509.0x speedup!

Slide 7 & 8: ModelSim Simulation & Waveform Verification

Speaking Script: Our SystemVerilog hardware modules were compiled and simulated in ModelSim using automated .do scripts (do run_sim.do). At 40 ns, Row 0 enters the accelerator. At 90 ns—exactly 5 clock cycles later—row_valid_out goes HIGH and the first 2D filtered row streams out. The testbench verified hardware outputs with 0 Errors.

Slide 9: Conclusion & Viva Defense

Speaking Script: In conclusion, PixelFlow proves that row-by-row feed-forward systolic processing eliminates line buffer bottlenecks, achieving ultra-low initial latency of 5 cycles and up to 509x speedup for high-resolution real-time image processing on FPGAs. Thank you! We are now open for questions and discussion.

PART 5: Viva Examiner Questions & Defense Answers (Q&A)

Q1: Why did you choose 8-bit input pixels and 20-bit intermediate registers?

Defense Answer: 8 bits per pixel is standard depth for Grayscale and RGB color channels (256 intensity levels). Filter weights are 16-bit signed integers to represent negative spatial derivatives ([-1, +3, -1]). When multiplying 8-bit pixels by 16-bit weights and accumulating 3 taps, max sum is ~2,295. A 20-bit signed register (-524,288 to +524,287) completely prevents intermediate arithmetic overflow while keeping hardware area minimal.

Q2: What is the difference between Direct-Form and Transpose-Form FIR filters?

Defense Answer: In Direct-Form (used in RConv), delay registers are placed at the input stream before multipliers. It computes row dot-products in parallel in 1 cycle with 0 line delays. In Transpose-Form (used in CConv), multipliers feed directly into line delay registers Z^-N placed between stages. This allows intermediate column partial sums to accumulate across rows as data streams through, producing the first output row after only 5 cycles.

Q3: How do you handle image boundary pixels (Zero Padding)?

Defense Answer: Inside rconv_array.v, we instantiate boundary padding logic: padded_row[0] = 0 (left pad), padded_row[1..N] = input_pixels, and padded_row[N+1] = 0 (right pad). This zero-padding guarantees that 3x3 filtering at image edges does not produce out-of-bounds memory errors.

Q4: How is the ModelSim waveform verified?

Defense Answer: We wrote a self-checking SystemVerilog testbench (tb_image_filter.v). It loads hex memory data via $readmemh, measures cycle counts between start and row_valid_out, compares hardware output against golden software reference vectors, and reports $finish with 0 Errors.