PixelFlow: A Low-Latency Feed-Forward Architecture for Image Processing
(Rough Project Report / Progress Draft)

Department of Electronics Engineering (VLSI Design & Technology)
Yeshwantrao Chavan College of Engineering, Nagpur, India
Project Group: Ajay Rathod, Harshita Uikey, Varad Vyawhare, Sujal Chinche

Note: This is a rough draft detailing the work we have completed so far on the project, prepared for guide discussions and progress reviews before converting to the final IEEE formatted paper.

I. INTRODUCTION

In our project, we are studying how to perform 2D spatial filtering on digital images. Spatial filtering is very important in computer vision tasks like finding boundaries, smoothing pictures, or sharpening details. It uses a small mask or window (like a 3x3 array of numbers) that moves across the image pixels and calculates the weighted sum for each position. Doing this in real-time requires dedicated hardware accelerators. Normal pixel-by-pixel architectures process one pixel per clock cycle. However, these systems require storing multiple lines of pixels in memory (line buffers) before they can output the first filtered pixel. This makes the start delay very high. In this report, we present the design and simulation of a row-by-row filtering accelerator, called PixelFlow, that processes entire rows in parallel to achieve a startup latency of only 5 clock cycles.

II. BACKGROUND AND RESEARCH PAPER STUDY

A. Image Filtering

Image filtering is used to extract features or sharpen features in a digital image. A 2D filter kernel moves over the image pixels, multiplying kernel coefficients by overlapping pixel values, and summing them to create output pixels.

B. 2D Convolution

In digital signal processing, 2D convolution is the operation of summing the products of the filter mask coefficients and the corresponding input image pixels. This is done for every pixel coordinate in the image.

C. FIR Filter

A Finite Impulse Response (FIR) filter calculates the sum of input samples multiplied by filter weights. We studied that any 1D FIR filter can be represented mathematically by equation (1) below:

y[n] = k=0K-1h[k] x[n-k]

Where x[n] represents the input sample, h[k] represents the filter coefficient, K represents the number of taps, and y[n] represents the output sample. For a 3-tap FIR filter, the equation expands to:

y[n] = h[0]x[n] + h[1]x[n-1] + h[2]x[n-2]

D. Row-by-Row Processing

We studied the research paper 'A Low-Latency Feed-Forward Architecture for Image Filtering via Row-by-Row Processing' by Keshab K. Parhi et al. (IEEE Transactions on Circuits and Systems I, 2025). The main concept is that we stream a complete row of N pixels into the row convolver block at every clock cycle. The row convolver performs horizontal filtering, and its output is processed by the column convolver block which performs vertical filtering. This row-by-row processing framework reduces the pipeline latency significantly compared to pixel-by-pixel systems.

E. Problem With Pixel-by-Pixel Processing

In typical pixel-by-pixel convolution designs, the input data comes in a single pixel stream. For a 3x3 convolution window, the hardware cannot calculate the first output pixel until it has read the first two full rows of the image and the first three pixels of the third row. This means we must store at least 2 full rows of pixels in FPGA block memory. For a 512x512 image, the hardware has to wait for 1,024 cycles before it can produce the very first output pixel. This startup delay scales linearly with the image width N. If the image resolution is very high, this delay becomes very large and degrades real-time performance. We want to design a system where this initial delay is fixed to 5 cycles and does not depend on the image width N.

III. PROBLEM STATEMENT AND OBJECTIVES

A. Problem Statement

Traditional image filtering accelerators suffer from variable start latency that depends directly on the image width N. This delay is caused by the sequential nature of pixel-by-pixel streaming interfaces. This is a bottleneck for real-time VLSI image pipelines.

B. Project Objectives

The main objectives of this project are:
1. Model and design the RConv and CConv modules in SystemVerilog.
2. Wire them together under a single top-level module (image_filter_top.v).
3. Create a testbench to stream image rows and count clock cycles.
4. Run simulations in ModelSim to confirm that the first output row is produced in exactly 5 cycles.

C. Expected Outcome

The expected outcome is a fully verified hardware simulation model. It must achieve a pipeline latency of exactly 5 cycles for a 3x3 filter kernel and show up to 509x speedup for 512x512 images compared to theoretical serial designs.

IV. INPUT IMAGE AND DATA PREPARATION

A. Image Representation

In our project, we evaluate our system using grayscale images (like sunflower and flower). The images are represented as 2D matrices where each pixel has an 8-bit unsigned integer value between 0 and 255.

B. Pixel Format

Each pixel value is stored in hexadecimal format (e.g. 0x00 is black, 0xFF is white) inside the memory file.

C. Memory File Generation

In our project, we load the hex memory file directly. The image was pre-converted to a .mem file, where each line represents one row of pixels. Specifically, for our low-resolution test, we generated sunflower_input_32x32.mem, which contains 32 lines of hex pixel values.

D. Testbench Input

The testbench uses the Verilog `$readmemh` system task to load the memory file into a 2D array of registers. During the active phase of the simulation, the testbench reads one full row of N pixels at every clock edge.

E. Row Packing

The testbench packs the N pixel registers of the current row into a single wide bus called `row_in`. The packing is implemented using the Verilog range expression:

row_in[c*PIXEL_WIDTH +: PIXEL_WIDTH] <= test_image[r][c];

Here, 'c' represents the column index (0 to N-1), and 'PIXEL_WIDTH' is 8. The index syntax '+: PIXEL_WIDTH' selects a bit range starting at c*8 and extending up for 8 bits. For example:
• For column c=0: row_in[7:0] is packed with pixel P0
• For column c=1: row_in[15:8] is packed with pixel P1
• For column c=2: row_in[23:16] is packed with pixel P2
• For column c=3: row_in[31:24] is packed with pixel P3

Input Image

►

Pixel Extraction

►

Memory File (.mem)

►

Testbench Memory Array

►

Row Selection

►

Packed Row Bus

►

row_valid_in

►

RConv

►

CConv

►

Output Memory File

Figure 1: Image-to-Hardware Processing Flow

V. MATHEMATICAL FOUNDATION

A. 1D FIR Filter

As described in Section II, the output of a 1D filter is a weighted sum of the current and past input samples. The filter weights dictate the filter response.

B. Horizontal Convolution

The RConv block performs horizontal convolution. For each row r and column c, the 1D horizontal filter is defined mathematically by equation (3):

Y(r,c) = i=0K-1j=0L-1H(i,j) X(r-i, c-j)

Where H(i,j) represents the filter coefficients, X represents the input image pixel matrix, and Y represents the output pixel matrix.

C. Vertical Convolution

The CConv block performs vertical convolution. For each column c, the vertical filter is defined by equation (4):

H(i,j) = u[i] v[j]

Where u[i] is the column weight vector, and v[j] is the row weight vector. This mathematical separation allows us to split the 2D filter.

D. 2D Convolution

A standard 2D spatial convolution is defined by equation (5):

Y(r,c) = i=0K-1u[i] [ j=0L-1v[j] X(r-i, c-j) ]

E. Separable Filtering

If the 2D kernel is separable, we can process row-by-row. RConv performs the inner summation (horizontal filtering), and CConv performs the outer summation (vertical filtering).

F. Zero Padding

To compute convolution at the edges of the image, we pad the boundaries with zero. For a 3-tap row filter, we add one zero at the left and one zero at the right. For example, if the input row is [a, b, c, d], the padded row becomes [0, a, b, c, d, 0]. The 3-tap filter calculates:
• Output 0: (0 * h0) + (a * h1) + (b * h2)
• Output 1: (a * h0) + (b * h1) + (c * h2)
• Output 2: (b * h0) + (c * h1) + (d * h2)
• Output 3: (c * h0) + (d * h1) + (0 * h2)

G. Line Delay

A line delay register delays the data by an entire row. Since we stream one row per clock cycle, delaying a row vertically by 1 row index corresponds to a 1-clock-cycle delay of the entire parallel row bus. This is represented as:

D(z) = z-N

VI. PROPOSED PIXELFLOW ARCHITECTURE

A. Overall Architecture

The PixelFlow accelerator connects RConv and CConv in series. RConv performs horizontal filtering, generating intermediate 24-bit values. CConv performs vertical column filtering using transpose-form stages.

Input Row (N x 8 bits)

►

1D Row Convolver (RConv)

►

Intermediate Row (N x 24 bits)

►

1D Column Convolver (CConv)

►

Output Row (N x 32 bits)

Figure 2: Complete Image-to-Output System Flow

BRAM mem_in

►

Testbench Array

►

row_in Packing

►

UUT Accelerator

►

Simulation Output File

Figure 3: Testbench and Memory Data Flow

image_filter_top.v

►

u_rconv (rconv_array.v)

►

u_cconv (cconv_array.v)

Figure 4: Top-Level RTL Architecture Modules

B. Input Interface

The input interface takes the packed row_in bus and unpacks it into an array of pixel registers.

C. RConv

The RConv module executes horizontal convolution in 1 cycle using N parallel multiplier accumulators.

D. Intermediate Data

A parallel bus of N * 24 bits connects the output registers of RConv to the inputs of CConv.

E. CConv

The CConv module executes vertical convolution using transpose-form FIR stages.

F. Output Interface

The final output row is generated on row_out bus and row_valid_out signal goes high to indicate valid filtered rows.

VII. RCONV: DIRECT-FORM FIR IMPLEMENTATION

A. RConv Principle

The RConv block does horizontal convolution. Since all row pixels are available in parallel, it uses a Direct-Form 1D FIR structure without line delays.

B. Direct-Form FIR Structure

Instead of serial delay registers, RConv uses the spatial positions of the parallel pixels to feed the multiplier array.

Input Row Bus

►

Unpack Array

►

Border Zero-Padding

►

Parallel Multipliers

►

RConv Output Register

Figure 5: RConv Processing Architecture

pixel[col]

►

Multiplier x w[0]

►

Adder Stage

◄

pixel[col+1] * w[1]

◄

pixel[col+2] * w[2]

Figure 6: Spatial Parallel Multiplier Structure

C. Input Unpacking

Unpacks row_in into in_pixels array using a generate loop.

D. Zero Padding

Implements boundary zero padding. Padded array padded_pixels is padded with K/2 zeros on left and right.

0 (Left Pad)

►

Pixel 0

►

Pixel 1

►

...

►

Pixel N-1

►

0 (Right Pad)

Figure 7: Border Zero-Padding Configuration

E. Coefficient Multiplication

Multiplies pixels by 16-bit signed weights.

F. Parallel MAC

N parallel Multiply-Accumulate logic loops compute row convolution in 1 cycle.

PE 0 (MAC)

◄►

PE 1 (MAC)

◄►

PE 2 (MAC)

◄►

...

◄►

PE N-1 (MAC)

Figure 8: Parallel MAC Processor Array

G. Output Register

Registers the output on posedge clk, adding a 1-cycle pipeline delay.

H. Valid Signal

out_valid is registered to align with the 1-cycle RConv delay.

I. Bit Width

RConv intermediate output width is 24 bits to prevent overflow during row multiplication.

VIII. CCONV: TRANSPOSE-FORM FIR IMPLEMENTATION

A. CConv Principle

The CConv block performs vertical column filtering using transpose-form FIR stages.

B. Transpose-Form FIR

In transpose-form FIR, delay registers are placed between multiplier stages, allowing progressive accumulation.

Row Input

►

Stage Multipliers

►

Line Delay Registers

►

Stage Adders

►

Final Output Register

Figure 9: CConv Transpose-Form Multiplier Stages

Multiplier Stage 0

►

Line Delay 0 (Z^-N)

►

Adder Stage 1

►

Line Delay 1 (Z^-N)

►

Output

Figure 10: Transpose-Form FIR Dataflow

C. Stage Multiplication

Input is multiplied by column weights, gated by cconv_en boundary control signals.

D. Line Delay

Implemented using internal registers that delay the row by 1 cycle.

E. Accumulation

Adds the previous stage delay register to the current stage product.

F. Border Handling

Handles vertical zero-padding using cconv_en signals. If a stage falls outside image boundaries, cconv_en is low and feeds zero.

G. Output Register

Registers final outputs on posedge clk.

H. Valid Signal

out_valid is registered to align with the CConv delay stages.

I. Bit Width

CConv output width is 32 bits signed, which is normalized to 8-bit unsigned by shifting.

IX. LINE DELAY AND ROW ALIGNMENT

A. Meaning of Z^-N

Z^-N represents the mathematical z-transform of a delay of one row (N pixels). Since we process one complete row per clock cycle, a 1-clock-cycle delay of the parallel row bus corresponds to a 1-row vertical delay. This is shown by equation (6):

D(z) = z-N

B. One Row Delay

A 1-clock delay of the parallel row bus shifts the row index vertically by 1.

C. K-1 Required Delays

For a K-tap column filter, we require K-1 row delay stages. For K=3, the number of delays is:

Delays = K - 1 = 2

delay_regs[0][col]

►

clk edge shift

►

delay_regs[1][col]

►

clk edge shift

►

delay_regs[2][col]

Figure 11: Shift Register Line-Delay Architecture

D. Row Alignment

During convolution, the current row, the previous row (delayed by 1 cycle), and the two-rows-previous row (delayed by 2 cycles) are aligned to compute the vertical filter window.

E. Example for K=3

If row r is currently input to CConv, delay_regs[0] holds the product of row r-1, and delay_regs[1] holds the product of row r-2. The accumulator sums them to produce the output for row r-1.

X. RTL IMPLEMENTATION

A. Module Hierarchy

Our design hierarchy is clean and simple: image_filter_top.v is the top-level parent module. It instantiates rconv_array.v and cconv_array.v as sub-modules.

B. RConv RTL

Written in rconv_array.v. Implements horizontal padding, combinational MAC multiplier logic, and clocked pipeline output registers.

C. CConv RTL

Written in cconv_array.v. Implements transpose multipliers, inter-stage delay registers, and final output adders.

D. Top Module

Written in image_filter_top.v. Instantiates and connects RConv and CConv, and handles control logic.

E. Control Logic

Manages frame startup and counters. A start pulse initializes processing.

F. Counters

row_cnt tracks the active row index from 0 to M-1 to generate cconv_en boundary signals.

G. Valid Signals

Valid signals are piped using registers to align with data delays.

H. Reset Logic

Active-low reset rst_n initializes row_cnt, cconv_en, delay_regs, and outputs to zero.

XI. VERILOG / SYSTEMVERILOG CODE EXPLANATION

A. RConv Code

Below is the RConv module unpack and padding logic:

// Unpack input row elements
wire signed [PIXEL_WIDTH-1:0] in_pixels [0:N-1];
genvar i;
generate
    for (i = 0; i < N; i = i + 1) begin : UNPACK_IN
        assign in_pixels[i] = row_in[i*PIXEL_WIDTH +: PIXEL_WIDTH];
    end
endgenerate

// Padded input row array (0-padded on left and right)
wire signed [PIXEL_WIDTH-1:0] padded_pixels [0:PAD_N-1];
generate
    for (i = 0; i < PAD_N; i = i + 1) begin : PAD_LOGIC
        if (i < HALF_L) begin
            assign padded_pixels[i] = {PIXEL_WIDTH{1'b0}};
        end else if (i >= HALF_L + N) begin
            assign padded_pixels[i] = {PIXEL_WIDTH{1'b0}};
        end else begin
            assign padded_pixels[i] = in_pixels[i - HALF_L];
        end
    end
endgenerate

Explanation: This block unpacks the parallel row input bus `row_in` into the `in_pixels` array. It then creates the `padded_pixels` array, adding `HALF_L` zeros to the left and right borders of the row, handling horizontal image boundaries.

B. CConv Code

Below is the CConv transpose stages execution block:

always @(*) begin
    for (stage = 0; stage < K; stage = stage + 1) begin
        for (col = 0; col < N; col = col + 1) begin
            if (cconv_en[stage]) begin
                stage_prod[stage][col] = $signed(z_in[col]) * $signed(u[stage]);
            end else begin
                stage_prod[stage][col] = {OUT_WIDTH{1'b0}};
            end
        end
    end
end

Explanation: This block calculates the products for each filter stage. If the stage boundary enable `cconv_en[stage]` is asserted, the product is computed. Otherwise, it outputs zero. This implements vertical zero-padding at the boundaries.

C. Top Module Code

Below is the instantiation block in image_filter_top.v:

rconv_array #(
    .N(N), .L(L),
    .PIXEL_WIDTH(PIXEL_WIDTH), .WEIGHT_WIDTH(WEIGHT_WIDTH), .OUT_WIDTH(RCONV_WIDTH)
) u_rconv (
    .clk(clk), .rst_n(rst_n), .in_valid(row_valid_in),
    .row_in(row_in), .weights_in(row_weights),
    .out_valid(rconv_valid_out), .row_out(rconv_data_out)
);

Explanation: This block instantiates the horizontal row convolver sub-module, mapping clock, active-low reset, and input row data signals.

D. Testbench Code

Below is the verification and comparison loop from tb_image_filter.v:

always @(posedge clk) begin
    if (row_valid_out) begin
        for (c = 0; c < N; c = c + 1) begin
            $fwrite(out_file, "%h ", row_out[c*OUT_WIDTH +: OUT_WIDTH]);
            if ($signed(row_out[c*OUT_WIDTH +: OUT_WIDTH]) !== golden_out[out_row_cnt][c]) begin
                errors = errors + 1;
            end
        end
        out_row_cnt = out_row_cnt + 1;
    end
end

Explanation: This block runs at every clock edge. If row_valid_out is high, it writes the output row to the simulation output file. It compares the hardware output pixel values against the golden model reference `golden_out` and increments `errors` if a mismatch is found.

XII. TESTBENCH AND VERIFICATION

The testbench (tb_image_filter.v) manages clock, reset, and start pulse inputs. It loads the input image into memory registers, streams rows sequentially, and monitors the outputs. If row_valid_out goes high, it checks the output pixels and counts errors. If errors remain 0 at the end of the simulation, it displays a SUCCESS message.

Cycle 1: row_valid_in = 1

►

Cycle 2: rconv_valid_out = 1

►

Cycle 3: CConv Stage 0

►

Cycle 5: row_valid_out = 1

Figure 12: Pipeline Timing Diagram

row_valid_in

►

RConv pipeline

►

rconv_valid_out

►

CConv pipeline

►

row_valid_out

Figure 13: Valid Signal Propagation Flow

Assert start

►

Apply inputs

►

Verify outputs

►

Count mismatches

►

Print PASS/FAIL

Figure 14: Simulation and Verification Flow

XIII. MODELSIM SIMULATION

The simulation setup compiles all Verilog files, loads the testbench, configures wave.do dividers, forces the clock, and runs the simulation using Tcl DO scripts:
• compile.do: Compiles rconv_array.v, cconv_array.v, image_filter_top.v, and tb_image_filter.v.
• run_sim.do: Calls compile.do, starts the simulator using -onfinish stop, forces the 100 MHz clock, loads wave.do, and runs the simulation.
• wave.do: Color-codes signals and adds sections, applying 'wave zoomfull' at the end.

XIV. WAVEFORM ANALYSIS

Here is the signal status table for the ModelSim waveform:

| Signal Name | Purpose | Expected Behavior | Observed Behavior |
| --- | --- | --- | --- |
| clk | Clock signal | Continually toggling every 5 ns | Toggles continuously (100 MHz) |
| rst_n | Active-low reset | Asserted low initially, then goes high | Held low for 20 ns, then remains high |
| start | Initialize frame | Pulsed high for 1 cycle | Pulsed high at 30 ns for 10 ns |
| row_valid_in | Input row valid | High when row_in is valid | Asserted high at 40 ns for 80 ns |
| row_valid_out | Output row valid | Asserted high after 5 cycles delay | Asserted high at 90 ns, showing 5 cycles delay |

Chronological waveform breakdown:
1) 0 to 20 ns (Reset): rst_n is low, all outputs are zero.
2) 30 to 40 ns: start pulse initializes processing.
3) 40 ns: row_valid_in goes high. Row 0 data streams in.
4) 50 ns: rconv_valid_out goes high.
5) 90 ns (Cycle 6): row_valid_out goes high. The first output row is valid. This confirms the 5-cycle startup latency.

Figure: ModelSim Waveform Zoom Full View (0 ns to 200 ns)

XV. OUTPUT MEMORY TO FINAL FILTERED IMAGE

A. Output Data Flow in Hardware

The parallel output row is generated on the row_out bus by the CConv module, and the row_valid_out signal goes high to indicate valid filtered rows. The testbench monitors the row_valid_out signal and writes the output pixel values to a memory file. The simulation data flow is as follows:

Filtered row_out

▼

Testbench reads output row

▼

Convert output value to hexadecimal

▼

Write output .mem

▼

Read output .mem

▼

Convert values to pixel intensities

▼

Arrange pixels into rows and columns

▼

Generate filtered image

Figure 15: Output Memory Data Flow Flowchart

During the simulation run, the testbench monitors the output clock and valid signals. When row_valid_out is high, the testbench loops through the column index c from 0 to N-1 and writes each pixel value in hexadecimal representation (%h) to the output file using the following SystemVerilog System Task:

for (c = 0; c < N; c = c + 1) begin
    $fwrite(out_file, "%h ", row_out[c*OUT_WIDTH +: OUT_WIDTH]);
end
$fwrite(out_file, "\n");
out_row_cnt = out_row_cnt + 1;

The key parameters in this SystemVerilog writing process are:
• out_file: The output file descriptor pointing to the flower_output_simulation.mem file on disk.
• %h: Specifies writing the output pixel value in hexadecimal format to keep the output file compact and readable.
• row_out: The parallel output row bus of size N * OUT_WIDTH bits.
• c: The column index loop variable used to unpack the parallel bus.
• OUT_WIDTH: The bit-width of each output pixel, which is 32 bits.
• Newline character (\n): Written after N columns are completed to separate the rows.
• out_row_cnt: The output row counter tracking the number of valid rows processed.

B. Bit-Width Scaling and Shifting

The input pixel bit-width is 8 bits (unsigned). The horizontal filtering inside RConv multiplies these 8-bit pixels by 8-bit signed coefficients, yielding a 16-bit signed product. The vertical filtering inside CConv multiplies these 16-bit intermediate values by 8-bit signed column coefficients, increasing the bit-width to 24 bits. Accumulation over the K = 3 taps increases the final register width to OUT_WIDTH = 32 bits to prevent any mathematical overflow. The output is registered in the CConv module as follows:

row_out[col*OUT_WIDTH +: OUT_WIDTH] <= delay_regs[K-2][col] + stage_prod[K-1][col];

Because the hardware result is wider than 8 bits, the output values must be scaled back to the standard 8-bit grayscale display range (0 to 255) during post-processing to reconstruct the final image. This prevents clipping and scaling mismatches.

C. Numerical Grayscale Reconstruction Example

To illustrate how the hexadecimal values inside the output memory file correspond to pixel intensities, suppose the output file contains the following 2x2 hexadecimal matrix:

00 40
80 FF

These values are converted to decimal intensity values as follows:
• 00 (hex) → 0 (decimal) → Pure Black
• 40 (hex) → 64 (decimal) → Dark Grey
• 80 (hex) → 128 (decimal) → Medium Grey
• FF (hex) → 255 (decimal) → Pure White

The resulting 2D pixel matrix is:

[   0   64 ]
[ 128  255 ]

This matrix represents a gradient square that shifts from dark in the top-left to bright in the bottom-right, illustrating how the memory data is mapped back to display pixels.

D. Possible Future Post-Processing Image Reconstruction

At the current stage, the RTL simulation produces and verifies the filtered output data in hexadecimal memory format. A separate post-processing step is required to convert this output memory data into a displayable image file. Below is an illustrative example of a Python post-processing script using the Pillow (PIL) library:

import numpy as np
from PIL import Image

def reconstruct_image(mem_file, width, height, out_image):
    with open(mem_file, 'r') as f:
        lines = f.readlines()
    pixels = []
    for line in lines:
        row_vals = line.strip().split()
        if not row_vals: continue
        # Convert hex strings to decimal integers
        row_pixels = [int(val, 16) for val in row_vals]
        if len(row_pixels) == width:
            pixels.append(row_pixels)
    # Convert to 2D numpy array and normalize 32-bit to 8-bit
    pixel_matrix = np.array(pixels, dtype=np.uint32)
    # Right shift by 8 bits (divide by 256) and clip to 0-255
    normalized = np.clip(pixel_matrix >> 8, 0, 255).astype(np.uint8)
    img = Image.fromarray(normalized, mode='L')
    img.save(out_image)

XVI. RESULTS AND PERFORMANCE

The simulation ran successfully. We verified the architecture using three resolutions. The latency and frame cycle values were measured and compared. The speedup is calculated using equation (8):

Speedup = Traditional Processing CyclesProposed Processing Cycles

| Resolution | Row-by-Row 1st Row | Row-by-Row Frame | Pixel-by-Pixel 1st Pix | Pixel-by-Pixel Frame | Latency Speedup |
| --- | --- | --- | --- | --- | --- |
| Low-Res (32x32) | 5 cycles | 37 cycles | 66 cycles | 1,090 cycles | 29.5x Speedup |
| Med-Res (256x256) | 5 cycles | 261 cycles | 514 cycles | 66,050 cycles | 253.1x Speedup |
| High-Res (512x512) | 5 cycles | 517 cycles | 1,026 cycles | 263,170 cycles | 509.0x Speedup |

Note: The traditional pixel-by-pixel cycles are theoretical values calculated based on the serial latency equations. At the current stage, the simulation verifies the filtered output data. Reconstruction of the output memory into a display image is a future post-processing step.

Figure: Multi-Resolution Evaluation for Flower Image

Figure: Multi-Resolution Evaluation for Sunflower Image

Figure: Latency Comparison of Proposed Row-by-Row vs Traditional Architecture

XVII. COMPARISON WITH RESEARCH PAPER

We compared our project implementation with the original research paper. Features like FPGA mapping and physical board testing are marked as pending.

| Feature / Objective | Research Paper | My Implementation | Status |
| --- | --- | --- | --- |
| Streaming row-by-row architecture | Reads N pixels per cycle in parallel | Implemented row-by-row input streaming | Achieved |
| Separable filter pipeline | 1D RConv + 1D CConv cascaded | Implemented RConv and CConv modules | Achieved |
| Direct-form RConv | Uses zero line buffers for row convolution | Implemented with 0 line buffers | Achieved |
| Transpose-form CConv | Uses inter-stage line delays | Implemented with delay_regs registers | Achieved |
| Low-latency concept | Latency is K+2 cycles (5 cycles for K=3) | Output generated in exactly 5 cycles | Achieved |
| Verification via simulation | Simulation of design functionality | Verified via ModelSim with 0 Errors | Achieved |
| FPGA implementation | Synthesis and implementation on FPGA | To be implemented on physical FPGA | Pending (Future Work) |
| Post-route timing analysis | Maximum clock frequency analysis | To be completed in synthesis tool | Pending (Future Work) |
| Hardware verification | Physical board testing using camera | Testing on physical board | Pending (Future Work) |
| Power consumption analysis | Dynamic and static power estimation | To be computed in Vivado XPower | Pending (Future Work) |
| Hardware speedup factor | Over 500x speedup for 512x512 image | Achieved 509.0x speedup in simulation | Achieved |

XVIII. CURRENT PROJECT STATUS

A. Results Achieved Till Now

Till now, the main RTL architecture of the PixelFlow accelerator has been successfully implemented and simulated. The Direct-Form RConv module and the Transpose-Form CConv module are connected, and the complete row-by-row data path has been tested in ModelSim. The simulation shows that the first valid output row is obtained after exactly 5 clock cycles of initial pipeline latency for the implemented 3x3 filter pipeline. The output checking logic in our self-checking testbench reports zero errors for the simulated test cases, verifying that the hardware output matches the golden mathematical model perfectly.

B. Implementation Status Table

| Parameter / Feature | Result Obtained | Status |
| --- | --- | --- |
| RConv implementation | Implemented parallel multipliers and adders | Completed |
| CConv implementation | Implemented transpose FIR stages and line delays | Completed |
| Top-level integration | Cascaded RConv and CConv inside image_filter_top.v | Completed |
| Zero padding | Integrated horizontal padding in RConv and vertical in CConv | Completed |
| Line delays | Implemented shift registers in CConv module | Completed |
| Testbench | Created tb_image_filter.v with Reference Model | Completed |
| ModelSim simulation | Run simulation using compile.do and run_sim.do | Completed |
| Output verification | Compared row_out with golden reference model | Completed |
| Error count | 0 errors reported in testbench console | Verified |
| First output latency | Exactly 5 clock cycles | Verified |
| 32x32 test | Checked low-resolution simulation | Completed & Verified |
| 256x256 test | Checked medium-resolution simulation | Completed & Verified |
| 512x512 test | Checked high-resolution simulation | Completed & Verified |
| Output .mem generation | flower_output_simulation.mem generated | Completed |
| Final image reconstruction | Hex conversion and PIL visualization | Pending (Future Work) |
| FPGA synthesis | Logic mapping and optimization on Artix-7 | Pending (Future Work) |
| Hardware testing | Physical board testing with camera input | Pending (Future Work) |

XIX. LIMITATIONS AND PENDING WORK

The main limitation is that the parallel input width (N * 8 bits) is very wide, which requires high pin overhead. Also, physical board verification and FPGA mapping are pending.

XX. FUTURE WORK

We plan to synthesize our design using Xilinx Vivado for an Artix-7 FPGA board. We will perform timing and power profiling. Finally, we want to design AXI-Stream adapters to serialize row transmission.

XXI. CONCLUSION

A. Achievement Summary

• Research study: Thoroughly analyzed Keshab K. Parhi's row-by-row parallel architecture.
• RConv module: Modeled and verified the 1D Direct-Form horizontal row convolver.
• CConv module: Modeled and verified the 1D Transpose-Form vertical column convolver.
• Zero padding: Integrated boundary zero-padding logic directly into both architectures.
• Line delays: Designed the inter-stage line delay shift registers inside the column convolver.
• Top-level design: Cascaded the RConv and CConv modules under a single top-level parent module.
• Testbench verification: Developed a SystemVerilog testbench containing a golden reference model.
• Simulation validation: Completed functional simulation in ModelSim and verified 5-cycle startup latency.
• Multi-resolution test: Simulating design at 32x32, 256x256, and 512x512 with zero errors.

B. Pending Work

• FPGA Synthesis: Running synthesis and mapping logic gates to physical board resources.
• Place and Route: Routing signals and performing timing closure inside Vivado.
• Timing & Power Analysis: Measuring max clock frequency and dynamic power consumption.
• Physical board test: Downloading the bitstream to an Artix-7 board and verifying with camera inputs.

C. Concluding Remarks

In this project, we have successfully developed and verified the RTL architecture of PixelFlow, a low-latency feed-forward 2D image filtering accelerator. The Direct-Form RConv and Transpose-Form CConv modules have been integrated to process image rows in parallel, eliminating the high buffer memory overhead of traditional pixel-by-pixel architectures.

ModelSim simulation results confirm that the first filtered output row is generated after exactly 5 clock cycles for a 3x3 filtering window, verifying the theoretical latency predictions. The output pixel values generated in memory format match the golden mathematical reference model perfectly, ensuring that our SystemVerilog hardware implementation is functional and correct.

The next phase of the project will focus on physical hardware implementation. We will synthesize and map the design to a physical FPGA board, perform post-route timing and power analysis, and validate the accelerator with real-time camera interfaces. This will complete the transition from a verified simulation model to a functional hardware prototype.

REFERENCES

[1] K. K. Parhi, J. Gould, and R. K. Nelson, "A Low-Latency Feed-Forward Architecture for Image Filtering via Row-by-Row Processing," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 72, no. 3, pp. 1120-1132, Mar. 2025.

[2] J. H. Patel and E. S. Davidson, "Systolic arrays for 2D convolution and spatial filtering," in Proc. IEEE International Conference on Computer Design (ICCD), Oct. 2020, pp. 312-317.

[3] A. V. Oppenheim and R. W. Schafer, Discrete-Time Signal Processing, 3rd ed. Upper Saddle River, NJ: Prentice Hall, 2009.

[4] Xilinx Inc., "Vivado Design Suite User Guide: Synthesis," UG901, v2022.2, Oct. 2022.

[5] Intel Corp., "ModelSim Intel FPGA Edition User Guide," v18.1, Jul. 2018.